Amdahl’s Law –Problems with Solutions

Parallel and GPU Computing

Amdahl’s Law Formula

Speedup S(N) = 1 / [(1 − P) + (P / N)]

Where:

P = parallel portion of the program

1 − P = sequential portion of the program

N = number of processors

S(N) = theoretical speedup

Question 1

A parallel program consists of 75% parallelizable code and 25% sequential code. "
        "The program is executed on 4 processors. Using Amdahl’s Law, calculate:

a) The theoretical speedup achieved using 4 processors.

b) The execution time if the original execution time on one processor is 80 seconds.

c) Comment on why the speedup is less than 4.

Solution

Given:

P = 0.75

1 − P = 0.25

N = 4

Original execution time = 80 seconds

a) Theoretical speedup:

S(4) = 1 / [(1 − 0.75) + (0.75 / 4)]

= 1 / [0.25 + 0.1875]

= 1 / 0.4375

≈ 2.29

Therefore, theoretical speedup ≈ 2.29×.

b) New execution time:

Tnew = Told / Speedup

= 80 / 2.29

≈ 34.91 seconds

Therefore, new execution time ≈ 34.9 seconds.

c) Why is the speedup less than 4?

Although 4 processors are used, 25% of the program is sequential and cannot be divided among the processors. This sequential portion limits the maximum speedup. Therefore, a 4× speedup cannot be achieved.

Question 2

A program takes 120 seconds to execute on a single processor. Out of the total execution time, 80% of the program can be parallelized, while the remaining 20% is sequential. Using Amdahl’s Law, calculate:

a) The theoretical speedup when 8 processors are used.

b) The new execution time with 8 processors.

c) What is the maximum possible speedup if an unlimited number of processors are used?

Solution

Given:

Original execution time = 120 seconds

P = 0.80

1 − P = 0.20

N = 8

a) Theoretical speedup:

S(8) = 1 / [(1 − 0.80) + (0.80 / 8)]

= 1 / [0.20 + 0.10]

= 1 / 0.30

= 3.33

Therefore, theoretical speedup ≈ 3.33×.

b) New execution time:

Tnew = Told / Speedup

= 120 / 3.33

≈ 36.04 seconds

Using the exact speedup of 10/3:

Tnew = 120 / (10/3)

= 36 seconds

Therefore, new execution time = 36 seconds.

c) Maximum possible speedup:

If N approaches infinity, P/N approaches 0.

Smax = 1 / (1 − P)

= 1 / 0.20

= 5

Therefore, the maximum theoretical speedup is 5×.

Question 3

Consider a program in which 15% of the execution is sequential and the remaining 85% can be executed in parallel.

a) Calculate the theoretical speedup when the program runs on 5 processors using Amdahl’s Law.

b) Calculate the theoretical speedup when 10 processors are used.

c) Compare the two results and explain why doubling the number of processors does not double the speedup.

Solution

Given:

Sequential portion = 0.15

Parallel portion P = 0.85

a) Speedup with 5 processors:

S(5) = 1 / [0.15 + (0.85 / 5)]

= 1 / [0.15 + 0.17]

= 1 / 0.32

= 3.125

Therefore, speedup with 5 processors = 3.125×.

b) Speedup with 10 processors:

S(10) = 1 / [0.15 + (0.85 / 10)]

= 1 / [0.15 + 0.085]

= 1 / 0.235

≈ 4.26

Therefore, speedup with 10 processors ≈ 4.26×.

c) Comparison:

With 5 processors → 3.125× speedup

With 10 processors → 4.26× speedup

The number of processors is doubled from 5 to 10, but the speedup does not double because 15% of the program is sequential. This sequential portion cannot benefit from additional processors and limits the overall speedup.